Platform Stabilization with 3DOF IMU Using PWM and PID

Introduction:

Our stabilising platform using PWM and PD, uses an IMU sensor, feedback control, and real-time actuation. The core idea behind the project is: when the platform is disturbed, either tilted or rotated due to external forces, it should actively counteract the motion and return itself to a stable, level orientation. This concept is similar to the balancing mechanisms used in drones, robotic arms, camera gimbals, and other mechatronic systems that require fast and precise stabilization.

In this project, a two-axis stabilization platform was developed using an Inertial Measurement Unit (IMU), two servo motors, and a Texas Instruments Tiva C microcontroller. The IMU continuously measures the platform’s pitch and roll angles through its onboard accelerometer. These raw measurements are then processed to estimate the real-time orientation of the system and are also displayed to track the movements on Docklight using UART. Once the Tiva C receives these angle readings, it computes the corrective actions required to maintain a level position. The servo motors then physically adjust the platform by rotating in the opposite direction of the detected tilt, completing the feedback control loop.

This project showcases the fundamental workflow of closed-loop control: sensing, computation, and actuation. The IMU rapid feedback about the platform’s dynamics. The microcontroller serves as the brain for our project, filtering the sensor data. The servos act as the muscles, executing corrections with precise angular displacement. Together, these components form a responsive system capable of compensating for disturbances in real time and maintaining a zero-degree horizontal position of the circular platform.

Hardware Setup

The hardware configuration of this self-stabilizing platform consists of three key components: the MPU6050 IMU sensor, the TM4C123GH6PM (Tiva C) microcontroller, and two servo motors responsible for stabilizing pitch and roll angles. These components form a continuous sensing–processing–actuation loop, enabling the platform to return to its reference orientation in real time, which is a zero degree horizontal position.

The MPU6050 (IMU) integrates a 3-axis accelerometer single I²C-based module. We do not use the gyroscope in the IMU because it is meant for relatively higher speed movements and gyroscope integration is unnecessary for relatively slow tilt corrections and avoids drift accumulation issues. This sensor is mounted on the servo motor attached directly to our handle to detect its instantaneous tilt. The accelerometer provides raw linear acceleration values along the X, Y, and Z axes.

The MPU6050 communicates with the Tiva C via the I²C protocol using two lines:

- SDA (Serial Data): connected to PB2

- SCL (Serial Clock): connected to PB3

These pins are configured with open-drain output and pull-up resistors to ensure proper I²C bus characteristics and voltage levels.

The Tiva C microcontroller serves as the central processing unit. It interfaces with the MPU6050 via I2C0, processes raw sensor data to compute orientation angles, executes the PID control algorithm, and generates PWM signals to drive the servo motors. Additionally, it transmits debugging information over UART0 via Port A (PA0 = RX, PA1 = TX) for real-time monitoring on a serial terminal.

The microcontroller operates at a system clock frequency that is prescaled to generate a 3.2 MHz PWM clock for precise servo control.

Two servo motors provide actuation for the pitch and roll axes. Each servo requires a standard 50 Hz PWM signal with pulse widths ranging from:

- 1 ms: corresponding to 0° position

- 2 ms: corresponding to 180° position

The servos are controlled by PWM outputs from the Tiva C:

- Roll servo: controlled via PWM1 Module 3, Generator 3 on PF2

- Pitch servo: controlled via PWM0 Module 1, Generator 1 on PB4

The servos physically adjust the platform's orientation in response to control signals, counteracting detected tilts and maintaining stability.

The MPU6050 is mounted on the servo motor attached to the handle to accurately measure its orientation. The servo motors are mechanically coupled to the platform structure, allowing independent control of pitch and roll axes. This physical arrangement ensures that sensor readings directly correspond to platform attitude and that servo actuation produces the intended corrective motion.

The handle: to make it user-friendly

The platform

Servo 2 attached to the platform via a connecting sheet

IMU sensor on servo 1

Tiva C microcontroller

Battery: to make it portable

Software Setup

The software implementation manages sensor initialization, data acquisition, signal processing, control algorithm execution, and actuator output generation. The code runs in an infinite loop on the Tiva C microcontroller, continuously maintaining platform stability.

I²C Communication Initialization

The I2C peripheral is initialized to communicate with the MPU6050. This involves:

- Enabling the I²C and GPIO Port B clocks via the system control registers

- Configuring PB2 (SDA) and PB3 (SCL) for alternate function mode

- Setting the peripheral control (PCTL) registers to select I²C functionality

- Enabling digital function and configuring PB3 (SDA) as open-drain

- Enabling pull-up resistors on both pins

The software includes I²C read and write functions (I2C0_Write, MPU_ReadBytes) that handle register-level communication with the MPU6050.

MPU6050 Sensor Configuration

The sensor is initialized through a series of register writes:

I2C0_Write(MPU_ADDR, 0x6B, 0x00);  // Enable

I2C0_Write(MPU_ADDR, 0x1A, 0x03);  // Configure low-pass filter

I2C0_Write(MPU_ADDR, 0x1B, 0x00);  // Set gyro range to ±250°/s

I2C0_Write(MPU_ADDR, 0x1C, 0x00);  // Set accelerometer range to ±2g

I2C0_Write(MPU_ADDR, 0x19, 0x07);  // Set sample rate divider

```

Raw accelerometer data is read from registers `0x3B` through `0x40` using the `MPU_ReadAccel()` function, which returns 16-bit signed integers for each axis.

Angle Calculation

Raw accelerometer values are first scaled to physical units (g-forces):

ax = Ax / 16384.0

ay = Ay / 16384.0

az = Az / 16384.0

The software then computes orientation angles using inverse trigonometric functions:

Roll angle (rotation around X-axis):

roll_acc = atan2(ay, az) × (180/π)

Pitch angle (rotation around Y-axis):

pitch_acc = atan2(-ax, √(ay² + az²)) × (180/π)

These instantaneous values are smoothed using a low-pass filter to reduce sensor noise:

filtered = prev_value + α(new - prev_value)

where α = 0.8.

UART Debugging Interface

UART0 is initialized for serial communication at 115200 baud:

- Enable UART0 and GPIO Port A clocks

- Configure PA0 and PA1 for alternate function (UART)

- Set baud rate divisors (IBRD = 325, FBRD = 33)

- Configure 8-bit data, no parity, 1 stop bit

- Enable UART transmitter and receiver

The software formats sensor readings, computed angles, and system status into strings using sprintf() and transmits them via UART0_WriteString() for real-time monitoring.

PWM Configuration

Two PWM channels are configured for servo control, both using identical timing parameters:

PWM1 Module 3 (Roll Servo on PF2):

SYSCTL->RCGCPWM |= 0x02;           // Enable PWM1 clock

GPIOF->AFSEL |= 0x04;              // PF2 alternate function

GPIOF->PCTL |= 0x00000500;         // Configure for PWM

PWM1->_3_GENA = 0x0000008C;        // Set on LOAD, clear on CMPA

PWM1->_3_LOAD = 62499;             // 20ms period

PWM1->_3_CMPA = 59999;             // Initial 1ms pulse

PWM1->_3_CTL = 0x01;               // Enable generator

PWM1->ENABLE |= 0x40;              // Enable output

PWM0 Module 1 (Pitch Servo on PB4):

SYSCTL->RCGCPWM |= 0x01;           // Enable PWM0 clock

GPIOB->AFSEL |= (1<<4);            // PB4 alternate function

GPIOB->PCTL |= 0x00040000;         // Configure for PWM

PWM0->_1_GENA = 0x0000008C;        // Set on LOAD, clear on CMPA

PWM0->_1_LOAD = 62499;             // 20ms period

PWM0->_1_CMPA = 59999;             // Initial 1ms pulse

PWM0->_1_CTL = 1;                  // Enable generator

PWM0->ENABLE |= 0x04;              // Enable output

The 20 ms period (50 Hz) is achieved by:

Period = (PWM_LOAD + 1) / PWM_Clock

Period = 62500 / 3,200,000 = 0.020 s

Servo Position Mapping

The RollToPWM() function converts control angles to PWM duty cycles using linear mapping:

CMPA = m × angle + b

where:

- m = -44.64 (slope)

- b = 57499 (intercept)

This maps control outputs in the range ±70° to CMPA values between 54374 and 60624, with clamping to prevent exceeding mechanical limits:

int cmpa = (int)(m * roll_angle + b);

if (cmpa > 60624) cmpa = 60624;

if (cmpa < 54374) cmpa = 54374;

PID Control Algorithm

The software implements a discrete-time PID controller for both axes. For each control cycle:

- Error calculation:

e = target - measured_angle

where target_roll = -10° and target_pitch = 16°

2. PID computation:

output = Kp·e + Ki·∫e dt + Kd·(de/dt)

with gains: Kp = 0.8, Ki = 0.0, Kd = 0.3

3. Derivative approximation:

derivative = error - previous_error

4. Integral accumulation:

integral += error

- Dead-band threshold:

if (fabs(output) < 1.0) output = 0;

- PWM update:

PWM1->_3_CMPA = RollToPWM(output_roll);

PWM0->_1_CMPA = RollToPWM(-output_pitch);

Note: The pitch output is negated to account for servo mounting orientation.

Freeze Mode Safety Logic

To prevent control instability during extreme tilts, the software implements a freeze mode:

if (pitch_f > 70.0f || pitch_f < -70.0f) {

power = 1;  // Activate freeze

} else if (pitch_f < 65.0f && pitch_f > -65.0f) {

power = 0;  // Deactivate freeze

}

When freeze mode is active (power = 1):

- Filtered angles are held at their last valid values

- PID integral terms are reset to zero

- Previous error terms are cleared

- Servos maintain their last commanded positions

This hysteresis prevents rapid mode switching and protects the system from aggressive control actions during recovery from extreme angles.

Main Control Loop

The infinite loop in main() executes the following sequence continuously:

- Read raw accelerometer data from MPU6050

- Convert to physical units (g-forces)

- Calculate instantaneous roll and pitch angles

- Apply low-pass filtering

- Check for freeze mode conditions

- Compute PID control outputs for both axes

- Convert control outputs to PWM duty cycles

- Update servo positions via PWM registers

- Transmit system status over UART

- Repeat

This closed-loop execution ensures continuous real-time stabilization, with typical loop rates determined by I²C communication speed and computation time.

Formulas Used

- ACCELEROMETER ANGLE CALCULATION:

Convert raw accelerometer values to units of g:
Ax = ax / 16384
Ay = ay / 16384
Az = az / 16384

Roll angle (degrees):
roll_acc = atan2(Ay, Az) × (180 / π)

Pitch angle (degrees):
pitch_acc = atan2(-Ax, sqrt(Ay² + Az²)) × (180 / π)

- LOW-PASS FILTERING (ANGLE SMOOTHING):

roll_f = roll_f + α(roll_acc – roll_f)

pitch_f = pitch_f + α(pitch_acc – pitch_f)

Where α = 0.8

- FREEZE MODE LOGIC:

If |pitch_f| > 70° → Enter freeze mode
If |pitch_f| < 65° → Exit freeze mode

When in freeze mode:
roll_f = previous_roll
PID integrals reset:
I_roll = 0
I_pitch = 0

- PD CONTROLLER EQUATIONS

Error:
e(t) = target – measured_angle

Proportional term:
P = Kp × e(t)

Derivative term:
D = Kd × (e(t) – e(t − 1))

Final control output:
u(t) = P + D

- PWM ANGLE MAPPING (ANGLE → SERVO PULSE)

CMPA = m × angle + b

Where:
m = (54374 – 60624) / (70 – (−70))
b = 57499

Clamp output:
If CMPA > 60624, set CMPA = 60624
If CMPA < 54374, set CMPA = 54374

Results

The stabilisation platform was tested under various conditions, with different objects of varying weights and sizes placed on the platform, at multiple angles to evaluate its ability to have to maintain target orientation of roll=10 degrees and pitch=16 degrees. The ideal value should have been 0 degrees for both, however, the hardware was slightly off-centred when assembling the hardware together and the angles were chosen to counteract the preexisting hardware disbalance. We assessed the performance  through real-time UART data logging and visual observation of the functionality.

Under normal operating conditions (pitch angles between -70° and +70°), the platform achieved steady-state stabilization within ±2° of the target angles. The roll axis converged to -10° ± 1.5°, while the pitch axis maintained 16° ± 2°. These deviations can be due to:

- Sensor noise in the MPU6050 accelerometer

- Mechanical backlash in servo linkages

- Discrete PWM resolution

- Quantization of filtered angle values to integers

Our platform successfully rejected continuous disturbances with active correction and returned to target and also catered to gradual angle changes compensated with minimal steady-state error.

To avoid the servos acting unpredictably at 90 degrees (it went to tan90 which is undefined), we froze the servos when pitch exceeded ±70°. Servos froze at last valid position within 50ms of threshold, PID integrators reset to prevent windup. Upon returning below 65°, normal control resumed smoothly.

We did observe some limitations during testing that need fine tuning:

- Accelerometer-only sensing fails near ±90°

- Servo response time (~60ms) limits maximum correction bandwidth

- Play between the servo motor and its attachment (the servo attached directly to the handle; we fixed the play caused by the second servo by replacing the attachment with one with finer stronger grooves).

We have tried using the gyroscope to fine tune the movements and better precision at angles near 90 degrees however that was not much different than how the accelerometer was performing, rather it made the movements too abrupt and let to objects on the platform falling off. Hence we decided to stick with the accelerometer for balanced moderate yet precise movements to counter the changes in balance.

| Group Members |  |
| --- | --- |
| Name | Roll Number |
| Ayma Aamir | 27100229 |
| Muhammad. | 27100221 |
| Zaeem Mohtashim Khan | 26100107 |